SO-101 Robotic Arm Lab

Session Report — March 9-10, 2026

Gopi Trinadh | University of Houston | CSE Research Assistant

# Executive Summary

Two-day intensive session building a custom robotics library for the SO-101 robotic arm from scratch. Fixed critical bugs in the LeRobot framework, built a two-layer control library (servo communication + arm management), created a dance demo, and established a complete development workflow.

# Day 1 Accomplishments (March 9)

## 1. LeRobot Bug Fixes (Windows)

The LeRobot framework from HuggingFace/Vizuara was failing on Windows during teleoperation. Through systematic debugging, we identified and fixed 5 separate issues:

| Bug | Root Cause | Fix |
| --- | --- | --- |
| No status packet | Return_Delay_Time=0 causing packet collisions | Increased to 2 (6µs delay) |
| Sync read failures | No retries on communication errors | Added num_retry=2 on sync_read |
| Torque enable crash | Rapid writes after configure_motors | Added 20ms delays + sleep after configure |
| Teleop crash on drop | ConnectionError kills entire loop | try/except to skip dropped frames |
| Gripper locks up | Overload protection too aggressive | Raised torque/current limits |

All fixes are committed to the Modern-Robot-Learning repo. These are Windows-specific issues caused by USB-serial driver timing differences between Linux and Windows.

## 2. Dance Demo

Created a 90-second choreographed dance performance for the follower arm. The routine includes 8 acts: awakening, gentle waves, exploring, building energy, climax with spiral motion, calming down, a bow, and return to rest. Recommended music: Hans Zimmer — "Time" (Inception). Script saved as so101_dance.py with command-line port argument.

## 3. Layer 1: ServoController (servo.py)

Built a complete servo communication library from scratch. This is the foundation that all higher layers build upon.

| Feature | Description |
| --- | --- |
| Individual Read/Write | Read position, speed, load, temperature, voltage per servo |
| Sync Read | Bulk read all 6 servos in one packet (2x faster, ~1090 Hz) |
| Speed Control | Presets: SLOW(200), MEDIUM(600), FAST(1500), MAX(0) |
| Acceleration Control | Presets: GENTLE(20), MEDIUM(80), FAST(200), INSTANT(254) |
| Smooth Move | Ease-in-out interpolation for natural movement |
| Torque Control | Enable/disable with safe goal=current before torque on |
| Benchmark | Compare individual vs sync read performance |
| Context Manager | 'with ServoController() as arm:' for safe cleanup |

# Day 2 Accomplishments (March 10)

## 4. Layer 2: ArmController (arm.py)

| Feature | Description |
| --- | --- |
| Port Detection | Unplug-based detection (like LeRobot's find-port method) |
| Saved Config | .ports.json saves detected ports for next time |
| Joint Limits | Safety clamping prevents arm from crashing into itself |
| Manual Calibration | Interactive joint-by-joint calibration |
| Quick Calibration | Move all joints, auto-track min/max range |
| Temperature Monitor | Color-coded display, auto-shutdown above 55°C |
| Emergency Stop | Instant torque kill on all servos |
| Status Display | Progress bars, temperatures, joint limits in one view |

## 5. CLI Entry Point (so101_cli.py)

Single command interface for all operations: so101_cli.py status, find-ports, calibrate, temps, benchmark, home. Replaces the need to remember which Python file to run.

## 6. Critical Bug Discovery — Servo Memory Corruption

During custom teleop testing, the follower arm exhibited violent uncontrolled movements. Root cause analysis revealed:

| Issue | Detail |
| --- | --- |
| Homing Offsets | LeRobot calibration wrote offset=5120 into all servo memory |
| Min Position Limits | Corrupted to 8224 (above max 4095) on multiple servos |
| Goal Position = 0 | After clearing offsets, old goal=0 + speed=0 (max speed) caused servos to race to position 0 |
| Resolution | Factory reset all servo memory: offset=0, min=0, max=4095. Re-calibrate via LeRobot. |

KEY LESSON: When enabling torque on STS3215 servos, ALWAYS set goal = current position FIRST. Otherwise the servo races to whatever old goal was stored in memory. This fix has been applied to servo.py's _set_torque method.

# Hardware Status

| Component | Status | Notes |
| --- | --- | --- |
| Leader Arm (COM5) | Working | All 6 servos responding, 24-25°C |
| Follower Arm (COM6) | Working | All 6 servos responding, 32-36°C |
| LeRobot Teleop | Working | 60Hz with Windows fixes applied |
| Custom Teleop | Working | 30Hz via raw scservo_sdk |
| Jetson Orin Nano | Ordered | Arriving soon, setup instructions prepared |
| USB Camera | To Buy | $8-12 generic 1080p USB webcam recommended |
| Intel RealSense | In Lab | Model TBD, wrist mount STL generated |

# Repository Structure

GitHub: github.com/GOPITRINADH3561/so101-robotic-arm-lab

so101-robotic-arm-lab/

├─ so101_cli.py           ← CLI entry point

├─ requirements.txt       ← pip dependencies

├─ .gitignore

├─ src/so101/

│   ├─ __init__.py        ← Package exports

│   ├─ servo.py           ← Layer 1: Servo communication

│   └─ arm.py             ← Layer 2: Arm controller

├─ assets/stl_models/     ← 3D printable parts

└─ docs/                  ← Assembly, BOM, configuration guides

# Jetson Orin Nano Setup (Tonight)

- Flash JetPack 6.x onto 128GB microSD using Balena Etcher

- Boot Jetson, complete Ubuntu setup (username, password, wifi)

- Install: sudo apt install -y git python3-pip python3-venv curl

- Install Miniforge (conda for ARM): curl -L -O https://github.com/conda-forge/miniforge/releases/latest/download/Miniforge3-Linux-aarch64.sh

- Create environment: conda create -n so101 python=3.11 -y

- Install: pip install feetech-servo-sdk numpy opencv-python pyserial

- Clone repo: git clone https://github.com/GOPITRINADH3561/so101-robotic-arm-lab.git

- USB permissions: sudo usermod -aG dialout $USER + udev rules + reboot

- Test: python src/so101/servo.py --port /dev/ttyUSB0 --test status

# Next Steps

| Priority | Task | Description |
| --- | --- | --- |
| Tomorrow | Layer 3: teleop.py | Custom teleoperation with safety, using ServoController |
| Tomorrow | Check RealSense | Identify model, measure wrist screw holes for mount |
| This Week | Layer 4: recorder.py | Record and replay demonstrations for imitation learning |
| This Week | Jetson Integration | Set up Jetson, test arm control on Linux |
| Next Week | Layer 5: vision.py | Camera + OpenCV integration, object detection |
| Next Week | Self-Calibration | Automatic range-of-motion detection (innovation feature) |
| Later | Layer 6: learner.py | Imitation learning using LeRobot's ACT/Diffusion policies |
| Later | LeKiWi Base | Mobile base integration (~$180 when budget allows) |

# Key Technical Learnings

- STS3215 servos store goal position, speed, offsets, and limits in non-volatile memory. Clearing these without setting proper defaults causes dangerous behavior.

- Speed=0 means MAXIMUM speed (no limit), not zero speed. Always set a positive speed value for controlled movement.

- Always set goal=current position BEFORE enabling torque. This prevents the servo from racing to an old goal position.

- Windows USB-serial drivers need more delay between packets than Linux. Adding 20ms pauses between writes fixes most communication issues.

- Sync read is 2x faster than individual reads (1090 Hz vs 542 Hz on Windows). Essential for real-time control loops.

- The feetech-servo-sdk package (PyPI) provides the scservo_sdk module. The package name and import name are different.

- LeRobot writes calibration data directly into servo EEPROM (homing offsets, min/max limits). Custom code must account for these values or reset them.

Generated: March 10, 2026 | SO-101 Robotic Arm Lab | University of Houston